ML525 MINI TRACK LOADER SPECIFICATIONS

DIMENSIONS

A. Angle of Departure ................................. 19°

B. Carry Position ........................................ 206 mm

C. Dump Angle @ Maximum Height ............ 36°

D. Dump Height with Standard Bucket ........ 1272 mm

E. Dump Reach @ Maximum Height ........... 815 mm

F. Ground Clearance .................................. 159 mm

G. Ground Clearance-under platform ........... 180 mm

H. Height to Bucket Hinge Pin .................... 1883 mm

I. Height ...................................................... 1352 mm

J. Length without Attachment ............................ 1976 mm

K. Length with Standard Bucket ......................... 2637 mm

L. Operating Height ....................................... 2649 mm

M. Rollback @ carry Position........................... 22°

N. Rollback Fully Raised

@ Maximum Height................................... 94°

O. Ground Contact Length ............................. 918 mm

P. Turning Radius with Standard Bucket ....... 1593 mm

Q. Width ........................................................ 1104 mm

R. Wheel Tread (over tracks) .......................... 904 mm

S. Width with 36" Bucket .................................. 1250 mm

PERFORMANCE

Operating Capacity (35%) ........................................................................................................................................ 291 kg

Operating Capacity (50%) ..................................................................................................................................416 kg

Tipping Capacity.................................................................................................................................................832 kg

Weight (no attachment) .....................................................................................................................................1060 kg

Travel Speed  .................................................................................................................................................... 5.6 km/h

ENGINE/ELECTRICAL

Make/Model ................................... Kubota // D1105-E4B-CSR-1

Fuel/Cooling ................................... Diesel/Liquid Horsepower (SAE Gross) ............... 18.5kW

Maximum Governed RPM .............. 3000 RPM

Torque @ 2200 RPM (SAE Net) ..... 71.5 Nm

Number of Cylinders	3

Displacement ................................. 1.123L

Bore/Stroke .................................... 78mm/78.4 mm

Fuel Consumption .......................... 6.1 L/h

Lubrication ..................................... Gear Pump Pressure Crankcase Ventilation .................... Closed

Air Cleaner ..................................... Dry replaceable cartridge with safety element

Ignition ........................................... Diesel-Compression

Engine Coolant ............................... Propylene glycol/water mix (53%-47%)

with freeze protection to -37°C

Starting Aid .................................... Glow plugs

Alternator ....................................... Belt Driven; 40 amps; Open

Battery ........................................... 12 V；45Ah

Starter ............................................ 12 volt；Gear Reduction Type; 1.4 kW

HYDRAULIC SYSTEM

Pump Type ............................................... Engine driven, Two Gear type

Pump Capacity ........................................ 53.4L/min@ 3000 RPM

System Relief @ Quick Couplers ............. 210 Bar

Hydraulic Filter ......................................... Full flow replaceable, 10 micron synthetic media element

Hydraulic Cylinders .................................. Double-acting

Main Control Valve ................................... 5-Spool, open center series parallel configuration

Attachment Control Valve…………………. 2-Spool, open center series parallel configuration

Bore Diameter

Lift Cylinder (2) .................................. 45mm

Tilt Cylinder (1) .................................. 55mm

Rod Diameter

Lift Cylinder (2) .................................. 25 mm

Tilt Cylinder (1) .................................. 30 mm

Stroke

Lift Cylinder (2) .................................. 295 mm

Tilt Cylinder (1) .................................. 280mm

Hydraulic Function Times

Raise Lift Arms .................................. 3.5 Seconds

Lower Lift Arms .................................. 2.4 Seconds

Bucket Dump ..................................... 2.5 Seconds

Bucket Rollback ................................. 1.8 Seconds

DRIVE SYSTEM

Main Drive .............. Fully hydraulic rubber track drive

Transmission .......... Direct drive of hydraulic motor to main undercarriage drive sprocket Tracks .................... 200 mm wide

Rollers……………..5 Each side

Pressure ………….. 25.3 kPa

CAPACITIES

Cooling System ..................................... 5.2 L

Fuel Tank ................................................ 35 L

Engine Oil with Filter .............................. 5.1L

Hydraulic Reservoir ............................ …34 L

Hydraulic System ............................... ….40 L

CONTROLS

Vehicle Steering ................... Direction and speed controlled by two handle

Lift & Tilt ........................ Controlled by one hand lever

Front Auxiliary (Std.) ...... Controlled by one hand lever

Auxiliary Pressure Release .. Fore and aft movement of hand lever after engine has been shut off. Engine ................................. Hand lever throttle: Key-type starter switch and shutdown

Starting Aid .......................... Glow Plugs – Activated by key switch

INSTRUMENTATION

The mini track loader conditions are monitored by a combination of gauges and warning lights in the operator’s line of sight that monitor the following functions. The system shall alert the operator of

monitored loader malfunctions by way of an visual warning lights.

Gauges	Warning Lights

Hour meter	Engine Oil Pressure

Fuel	Engine Coolant Temperature

System Voltage